# Remote Sensing – Plot-Level ET Estimation

## 1. Abstract

## Objectives -

- To estimate daily evapotranspiration (ET) for 30 plots using WaPOR data.

- Deal with spatial resolution and temporal interpolation challenges.

- Compare satellite-based ET with ground-truth water usage from meters.

### 2.1. WaPOR Data

- Source: FAO WaPOR Level-1 (Dekadal ET, 300m resolution).

- Units: mm/day (dekadal sums provided).

### 2.2. Plot Information

- 30 plots across different farm management types:

  - TPR (Transplanted Rice)

  - DSR (Direct Seeded Rice)

  - AWD (Alternate Wetting and Drying)

- Plots are smaller than pixel resolution (100–200 m in size or 2-5 acres).

Link to Sample plots - https://docs.google.com/spreadsheets/d/1UrMY4plAIWYo_-UX7tvCAZfd4BBbuUAYrSaKru6elbY/edit?usp=sharing

## 3. Methodology

### 3.1. Pixel-Plot Overlap Resolution

#### Problem:

WaPOR pixels (300m) are coarser than plot boundaries, causing multiple pixel overlaps.

(plot id = LA_PA_PA_HARPA_PLT_5052)

#### Solution: Area-weighted averaging

- Overlay plot polygons on WaPOR raster.

- Calculate intersection area of each pixel with plot using Shoelace Algorithm (Green’s Theorem) or GIS tools.

- Multiply pixel ET by area fraction and sum:

Output -

#### Validation:

- Area-weighted averaging is widely used in downscaling satellite data to field level.

- Similar approach used in:

  - FAO’s WaPOR methodology documentation:

https://wapor.apps.fao.org/home/WAPOR_Methodology

  - NASA Land Data Assimilation Systems (LDAS) - Downscaling

“For point or plot-scale validation, the average value of the intersecting pixels weighted by the fraction of coverage should be used.”

https://ldas.gsfc.nasa.gov/

  - QGIS Documentation – Zonal Statistics

https://docs.qgis.org/

### 3.2. Temporal Interpolation

#### Problem:

ET data is dekadal (e.g., 12 mm every 10 days) — coarse for daily water analysis.

#### Solution: Cubic Spline Interpolation

Instead of assuming constant ET/day = total/10 (which is unrealistic), cubic spline interpolation was applied to smooth values across time.

Cubic spline interpolation is a method to construct a smooth curve that passes through a series of known data points. Unlike linear or polynomial interpolation, a cubic spline stitches together piecewise cubic polynomials between each pair of points, ensuring smooth transitions at the joints.

Spline interpolation creates daily or finer-resolution estimates from coarse data while maintaining realistic trends.This method better captures temporal variability, aligns with weather-induced ET shifts.

Validation:

- Cubic splines are commonly used for temporal interpolation in environmental datasets.

- Used in ET studies like:

  - Zhang et al., 2017 – Journal of Hydrology

  - NASA EarthData and MODIS preprocessing pipelines.

(all values in mm/day)

## 4. Visualizations and Analysis

- Daily ET curves per plot.

- Boxplots comparing ET distributions across TPR, DSR, and AWD.

- Whisker plots to show variability across seasons or plots.

- Comparative bar charts showing peak vs. low ET periods.

### Tools Used:

- Python (Matplotlib, Seaborn, Scipy for interpolation)

SUM UP

Flowchart of logic

Link to the final data (1 JUNE 2024 - 1 JUNE 2025) : https://docs.google.com/spreadsheets/d/1BIZ1y6_kQP5UEn6RG4NCZykfGErjCuwx8VpyyTQAMPo/edit?usp=sharing

## 5. Code Snippets

### 5.1 Area weighted average

import rasterio

import geopandas as gpd

import numpy as np

from rasterio.features import rasterize

from rasterio.transform import from_bounds

from shapely.geometry import box, Polygon

from shapely.ops import unary_union

import pandas as pd

from typing import Dict, List, Tuple, Optional

import warnings

import matplotlib.pyplot as plt

from matplotlib.patches import Polygon as MPLPolygon

warnings.filterwarnings('ignore')

def shoelace_area(coordinates: List[Tuple[float, float]]) -> float:

"""

Calculate polygon area using the shoelace formula.

Parameters:

-----------

coordinates : List[Tuple[float, float]]

List of (x, y) coordinate tuples forming the polygon

Returns:

--------

float

Area of the polygon

"""

if len(coordinates) < 3:

return 0.0

# Ensure polygon is closed

if coordinates[0] != coordinates[-1]:

coordinates = coordinates + [coordinates[0]]

n = len(coordinates) - 1  # Exclude the duplicate closing point

area = 0.0

for i in range(n):

j = (i + 1) % n

area += coordinates[i][0] * coordinates[j][1]

area -= coordinates[j][0] * coordinates[i][1]

return abs(area) / 2.0

def get_pixel_coordinates(pixel_row: int, pixel_col: int, transform: rasterio.Affine) -> List[Tuple[float, float]]:

"""

Get the corner coordinates of a pixel.

Parameters:

-----------

pixel_row : int

Row index of the pixel

pixel_col : int

Column index of the pixel

transform : rasterio.Affine

Raster transform

Returns:

--------

List[Tuple[float, float]]

List of corner coordinates [(x, y), ...]

"""

# Get pixel bounds in geographic coordinates

left = transform * (pixel_col, pixel_row)

right = transform * (pixel_col + 1, pixel_row)

bottom_left = transform * (pixel_col, pixel_row + 1)

bottom_right = transform * (pixel_col + 1, pixel_row + 1)

# Return corners in order: top-left, top-right, bottom-right, bottom-left

return [

left,           # top-left

right,          # top-right

bottom_right,   # bottom-right

bottom_left     # bottom-left

]

def get_intersection_coordinates(plot_geometry, pixel_coords: List[Tuple[float, float]]) -> Optional[List[Tuple[float, float]]]:

"""

Get the coordinates of intersection between plot geometry and pixel.

Parameters:

-----------

plot_geometry : shapely.geometry

Plot geometry

pixel_coords : List[Tuple[float, float]]

Pixel corner coordinates

Returns:

--------

Optional[List[Tuple[float, float]]]

Intersection coordinates or None if no intersection

"""

try:

# Create pixel polygon

pixel_polygon = Polygon(pixel_coords)

# Calculate intersection

intersection = plot_geometry.intersection(pixel_polygon)

if intersection.is_empty:

return None

# Handle different geometry types

if hasattr(intersection, 'exterior'):

# Single polygon

return list(intersection.exterior.coords[:-1])  # Exclude duplicate last point

elif hasattr(intersection, 'geoms'):

# MultiPolygon or GeometryCollection

all_coords = []

for geom in intersection.geoms:

if hasattr(geom, 'exterior'):

all_coords.extend(list(geom.exterior.coords[:-1]))

return all_coords if all_coords else None

else:

return None

except Exception as e:

print(f"Error calculating intersection: {e}")

return None

def calculate_precise_area_weighted_et(geotiff_path: str, geojson_path: str,

plot_id_column: str = 'id') -> pd.DataFrame:

"""

Calculate area-weighted ET values using precise coordinate-based area calculation.

Parameters:

-----------

geotiff_path : str

Path to the GeoTIFF file containing ET values

geojson_path : str

Path to the GeoJSON file containing plot boundaries

plot_id_column : str

Column name in GeoJSON that contains plot identifiers

Returns:

--------

pd.DataFrame

DataFrame with plot IDs and their area-weighted ET values

"""

# Read the GeoJSON file

plots_gdf = gpd.read_file(geojson_path)

# Read the GeoTIFF file

with rasterio.open(geotiff_path) as src:

et_data = src.read(1)  # Read first band

transform = src.transform

crs = src.crs

nodata = src.nodata

# Ensure both datasets have the same CRS

if plots_gdf.crs != crs:

plots_gdf = plots_gdf.to_crs(crs)

results = []

for idx, plot in plots_gdf.iterrows():

plot_id = plot[plot_id_column] if plot_id_column in plot else idx

geometry = plot.geometry

print(f"Processing plot {plot_id}...")

# Get the bounding box of the plot

minx, miny, maxx, maxy = geometry.bounds

# Convert bounds to pixel coordinates

ul_col, ul_row = ~transform * (minx, maxy)  # Upper left

lr_col, lr_row = ~transform * (maxx, miny)  # Lower right

# Round to get pixel indices

ul_col, ul_row = int(np.floor(ul_col)), int(np.floor(ul_row))

lr_col, lr_row = int(np.ceil(lr_col)), int(np.ceil(lr_row))

# Ensure indices are within raster bounds

ul_col = max(0, ul_col)

ul_row = max(0, ul_row)

lr_col = min(et_data.shape[1], lr_col)

lr_row = min(et_data.shape[0], lr_row)

if ul_col >= lr_col or ul_row >= lr_row:

print(f"Warning: Plot {plot_id} is outside raster bounds")

results.append({

'plot_id': plot_id,

'area_weighted_et': np.nan,

'total_intersection_area': 0,

'pixel_count': 0,

'intersection_details': []

})

continue

# Store intersection details for each pixel

intersection_details = []

total_intersection_area = 0

# Process each pixel in the bounding box

for row in range(ul_row, lr_row):

for col in range(ul_col, lr_col):

# Get pixel value

pixel_value = et_data[row, col]

# Skip nodata pixels

if nodata is not None and pixel_value == nodata:

continue

if np.isnan(pixel_value):

continue

# Get pixel corner coordinates

pixel_coords = get_pixel_coordinates(row, col, transform)

# Get intersection coordinates

intersection_coords = get_intersection_coordinates(geometry, pixel_coords)

if intersection_coords and len(intersection_coords) >= 3:

# Calculate intersection area using shoelace formula

intersection_area = shoelace_area(intersection_coords)

if intersection_area > 0:

intersection_details.append({

'pixel_row': row,

'pixel_col': col,

'pixel_value': pixel_value,

'intersection_area': intersection_area,

'intersection_coords': intersection_coords,

'pixel_coords': pixel_coords

})

total_intersection_area += intersection_area

# Calculate area-weighted ET

if total_intersection_area > 0 and intersection_details:

area_weighted_sum = 0

# Calculate weights and weighted sum

for detail in intersection_details:

weight = detail['intersection_area'] / total_intersection_area

detail['weight'] = weight

area_weighted_sum += detail['pixel_value'] * weight

area_weighted_et = area_weighted_sum

else:

area_weighted_et = np.nan

results.append({

'plot_id': plot_id,

'area_weighted_et': area_weighted_et,

'total_intersection_area': total_intersection_area,

'pixel_count': len(intersection_details),

'intersection_details': intersection_details

})

# Create DataFrame with main results

main_results = pd.DataFrame([{

'plot_id': r['plot_id'],

'area_weighted_et': r['area_weighted_et'],

'total_intersection_area': r['total_intersection_area'],

'pixel_count': r['pixel_count']

} for r in results])

# Store detailed results for visualization

main_results._detailed_results = results

return main_results

def visualize_precise_plot_coverage(geotiff_path: str, geojson_path: str, plot_id: str,

results_df: pd.DataFrame, plot_id_column: str = 'id'):

"""

Visualize precise plot coverage with coordinate-based intersections.

"""

# Get detailed results

detailed_results = getattr(results_df, '_detailed_results', None)

if detailed_results is None:

print("No detailed results available. Please run calculate_precise_area_weighted_et first.")

return

# Find the specific plot results

plot_result = None

for result in detailed_results:

if str(result['plot_id']) == str(plot_id):

plot_result = result

break

if plot_result is None:

print(f"Plot {plot_id} not found in results.")

return

# Read data

plots_gdf = gpd.read_file(geojson_path)

plot = plots_gdf[plots_gdf[plot_id_column] == plot_id].iloc[0]

with rasterio.open(geotiff_path) as src:

et_data = src.read(1)

transform = src.transform

crs = src.crs

nodata = src.nodata

# Ensure same CRS

if plots_gdf.crs != crs:

plots_gdf = plots_gdf.to_crs(crs)

plot = plots_gdf[plots_gdf[plot_id_column] == plot_id].iloc[0]

# Create visualization

fig, (ax1, ax2, ax3) = plt.subplots(1, 3, figsize=(24, 8))

# Get plot bounds with buffer

minx, miny, maxx, maxy = plot.geometry.bounds

buffer = max((maxx - minx), (maxy - miny)) * 0.2

plot_minx, plot_miny = minx - buffer, miny - buffer

plot_maxx, plot_maxy = maxx + buffer, maxy + buffer

# Convert to pixel coordinates for cropping

ul_col, ul_row = ~transform * (plot_minx, plot_maxy)

lr_col, lr_row = ~transform * (plot_maxx, plot_miny)

ul_col, ul_row = int(np.floor(ul_col)), int(np.floor(ul_row))

lr_col, lr_row = int(np.ceil(lr_col)), int(np.ceil(lr_row))

ul_col = max(0, ul_col)

ul_row = max(0, ul_row)

lr_col = min(et_data.shape[1], lr_col)

lr_row = min(et_data.shape[0], lr_row)

# Create extent

extent = [plot_minx, plot_maxx, plot_miny, plot_maxy]

# Crop raster

subset = et_data[ul_row:lr_row, ul_col:lr_col]

subset_masked = np.copy(subset).astype(float)

if nodata is not None:

subset_masked[subset == nodata] = np.nan

# Plot 1: ET data with plot overlay

im1 = ax1.imshow(subset_masked, extent=extent, origin='upper', alpha=0.8, cmap='viridis')

# Plot plot boundary

if hasattr(plot.geometry, 'exterior'):

x, y = plot.geometry.exterior.xy

ax1.plot(x, y, color='red', linewidth=3, label=f'Plot {plot_id}')

ax1.fill(x, y, color='red', alpha=0.2)

# Add pixel grid

for row in range(ul_row, lr_row + 1):

for col in range(ul_col, lr_col + 1):

pixel_coords = get_pixel_coordinates(row, col, transform)

if pixel_coords:

pixel_x = [coord[0] for coord in pixel_coords] + [pixel_coords[0][0]]

pixel_y = [coord[1] for coord in pixel_coords] + [pixel_coords[0][1]]

ax1.plot(pixel_x, pixel_y, color='white', alpha=0.5, linewidth=1)

ax1.set_title(f'Plot {plot_id} - ET Data with Pixel Grid', fontsize=14)

ax1.set_xlabel('X Coordinate')

ax1.set_ylabel('Y Coordinate')

ax1.legend()

plt.colorbar(im1, ax=ax1, label='ET Value', fraction=0.046, pad=0.04)

# Plot 2: Intersection areas

ax2.set_xlim(plot_minx, plot_maxx)

ax2.set_ylim(plot_miny, plot_maxy)

# Plot plot boundary

if hasattr(plot.geometry, 'exterior'):

x, y = plot.geometry.exterior.xy

ax2.plot(x, y, color='blue', linewidth=3, label=f'Plot {plot_id}')

# Plot intersection areas with different colors based on weights

colors = plt.cm.Reds(np.linspace(0.3, 1.0, len(plot_result['intersection_details'])))

for i, detail in enumerate(plot_result['intersection_details']):

# Plot pixel boundary

pixel_coords = detail['pixel_coords']

pixel_x = [coord[0] for coord in pixel_coords] + [pixel_coords[0][0]]

pixel_y = [coord[1] for coord in pixel_coords] + [pixel_coords[0][1]]

ax2.plot(pixel_x, pixel_y, color='black', linewidth=1, alpha=0.5)

# Plot intersection area

intersection_coords = detail['intersection_coords']

if len(intersection_coords) >= 3:

intersection_x = [coord[0] for coord in intersection_coords]

intersection_y = [coord[1] for coord in intersection_coords]

# Create matplotlib polygon

polygon = MPLPolygon(list(zip(intersection_x, intersection_y)),

facecolor=colors[i], alpha=0.7, edgecolor='black', linewidth=1)

ax2.add_patch(polygon)

# Add weight text

centroid_x = np.mean(intersection_x)

centroid_y = np.mean(intersection_y)

ax2.text(centroid_x, centroid_y, f'{detail["weight"]:.3f}',

ha='center', va='center', fontsize=8, fontweight='bold',

bbox=dict(boxstyle='round,pad=0.2', facecolor='white', alpha=0.8))

ax2.set_title(f'Plot {plot_id} - Intersection Areas & Weights', fontsize=14)

ax2.set_xlabel('X Coordinate')

ax2.set_ylabel('Y Coordinate')

ax2.legend()

ax2.set_aspect('equal')

# Plot 3: Summary statistics

ax3.axis('off')

# Prepare summary data

summary_text = f"=== Plot {plot_id} Summary ===\n\n"

summary_text += f"Area-weighted ET: {plot_result['area_weighted_et']:.6f}\n"

summary_text += f"Total intersection area: {plot_result['total_intersection_area']:.4f}\n"

summary_text += f"Number of intersecting pixels: {plot_result['pixel_count']}\n\n"

summary_text += "Pixel Details:\n"

summary_text += f"{'Pixel':<8} {'ET Value':<10} {'Area':<10} {'Weight':<10}\n"

summary_text += f"{'-'*50}\n"

for i, detail in enumerate(plot_result['intersection_details']):

pixel_pos = f"({detail['pixel_row']},{detail['pixel_col']})"

summary_text += f"{pixel_pos:<8} {detail['pixel_value']:<10.4f} "

summary_text += f"{detail['intersection_area']:<10.4f} {detail['weight']:<10.4f}\n"

# Verification

total_weight = sum(detail['weight'] for detail in plot_result['intersection_details'])

summary_text += f"\nWeight sum: {total_weight:.6f} (should be 1.0)\n"

# Simple average for comparison

pixel_values = [detail['pixel_value'] for detail in plot_result['intersection_details']]

if pixel_values:

simple_avg = np.mean(pixel_values)

summary_text += f"Simple average ET: {simple_avg:.6f}\n"

summary_text += f"Difference: {abs(plot_result['area_weighted_et'] - simple_avg):.6f}"

ax3.text(0.05, 0.95, summary_text, transform=ax3.transAxes, fontsize=10,

verticalalignment='top', fontfamily='monospace',

bbox=dict(boxstyle='round,pad=0.5', facecolor='lightgray', alpha=0.8))

plt.tight_layout()

plt.show()

return fig

def compare_methods(geotiff_path: str, geojson_path: str, plot_id_column: str = 'id'):

"""

Compare the original and precise methods.

"""

print("Running original method...")

# Note: You'll need to import your original function

# original_results = calculate_area_weighted_et(geotiff_path, geojson_path, plot_id_column)

print("Running precise method...")

precise_results = calculate_precise_area_weighted_et(geotiff_path, geojson_path, plot_id_column)

print("\nPrecise method results:")

print(precise_results.head())

return precise_results

# Example usage

if __name__ == "__main__":

# File paths

geotiff_path = "/content/5052_01_04_25"

geojson_path = "/content/plot_5052.json"

print("Calculating precise area-weighted ET values...")

# Calculate using precise coordinate-based method

results = calculate_precise_area_weighted_et(

geotiff_path,

geojson_path,

plot_id_column='id'

)

# Display results

print("\nPrecise Results:")

print(results)

# Save results to CSV

results.to_csv('precise_area_weighted_et.csv', index=False)

print("\nResults saved to precise_area_weighted_et.csv!")

# Visualize specific plots

if len(results) > 0:

plot_ids = results['plot_id'].head(2).tolist()

print(f"\nCreating detailed visualizations for plots: {plot_ids}")

for plot_id in plot_ids:

print(f"Visualizing plot {plot_id}...")

visualize_precise_plot_coverage(geotiff_path, geojson_path, str(plot_id),

results, plot_id_column='id')

print("\nPrecise calculation complete!")

### 5.2 Temporal Interpolation (Cubic spline)

import pandas as pd

import numpy as np

from scipy.interpolate import CubicSpline

import matplotlib.pyplot as plt

import matplotlib.dates as mdates

from datetime import datetime

import warnings

import os

# Suppress warnings for cleaner output

warnings.filterwarnings('ignore')

def test_file_access():

"""Test if the Excel file is accessible"""

test_file = '/content/june-oct.xlsx'

print("="*60)

print("CHECKING FILE ACCESS...")

print("="*60)

if os.path.exists(test_file):

print(f"✓ File found: {test_file}")

return test_file

else:

print(f"✗ File not found: {test_file}")

# Check for other Excel files

try:

files = os.listdir('/content/')

excel_files = [f for f in files if f.endswith(('.xlsx', '.xls'))]

if excel_files:

print("\nAvailable Excel files in /content/:")

for f in excel_files:

print(f"  - {f}")

# Use the first Excel file found

return f'/content/{excel_files[0]}'

else:

print("No Excel files found in /content/")

print("Please upload your Excel file to the Colab environment")

return None

except:

print("Could not access /content/ directory")

return None

def load_farm_type_tables(file_path):

"""Load the three farm type tables (TPR, DSR, AWD) from Excel file."""

# Try to read from different sheet names

try:

df_raw = pd.read_excel(file_path, sheet_name='Sheet1', header=None)

except:

try:

df_raw = pd.read_excel(file_path, sheet_name='Sheet2', header=None)

except:

df_raw = pd.read_excel(file_path, header=None)

# Find table boundaries automatically

table_starts = []

for idx, row in df_raw.iterrows():

if any(str(val).lower().strip() == 'date' for val in row.values if pd.notna(val)):

table_starts.append(idx)

print(f"Found {len(table_starts)} table headers at rows: {[x+1 for x in table_starts]}")

# Define table configurations

farm_type_names = ['TPR', 'DSR', 'AWD']

farm_tables = {}

for i, start_row in enumerate(table_starts):

farm_type = farm_type_names[i] if i < len(farm_type_names) else f'Table_{i+1}'

# Determine end row

if i + 1 < len(table_starts):

end_row = table_starts[i + 1]

else:

end_row = len(df_raw)

for j in range(start_row + 1, len(df_raw)):

if df_raw.iloc[j].isna().all():

has_more_data = False

for k in range(j + 1, min(j + 5, len(df_raw))):

if not df_raw.iloc[k].isna().all():

has_more_data = True

break

if not has_more_data:

end_row = j

break

# Extract headers and data

headers = df_raw.iloc[start_row].values

headers = [str(h) if pd.notna(h) else f'col_{i}' for i, h in enumerate(headers)]

data_start = start_row + 1

data_rows = df_raw.iloc[data_start:end_row].copy()

data_rows = data_rows.dropna(how='all')

if not data_rows.empty:

data_rows.columns = headers[:len(data_rows.columns)]

table_df = process_farm_table(data_rows, farm_type)

if not table_df.empty:

farm_tables[farm_type] = table_df

print(f"✓ {farm_type}: {len(table_df)} dekadal periods, {len(table_df.columns)} farms")

return farm_tables

def process_farm_table(table_data, farm_type):

"""Process individual farm table."""

table_data = table_data.copy()

# Find date column

date_col = None

for col in table_data.columns:

if 'date' in str(col).lower():

date_col = col

break

if date_col is None:

date_col = table_data.columns[0]

# Convert date column

try:

table_data[date_col] = pd.to_datetime(table_data[date_col], errors='coerce')

except:

try:

# Handle Excel serial numbers

table_data[date_col] = pd.to_datetime(table_data[date_col], origin='1899-12-30', unit='D', errors='coerce')

except:

print(f"Warning: Could not convert dates for {farm_type}")

return pd.DataFrame()

# Remove rows with invalid dates

table_data = table_data.dropna(subset=[date_col])

if table_data.empty:

return pd.DataFrame()

# Set date as index

table_data.set_index(date_col, inplace=True)

# Convert other columns to numeric

numeric_columns = []

for col in table_data.columns:

if col != date_col:

try:

if isinstance(table_data[col], pd.Series):

table_data[col] = pd.to_numeric(table_data[col], errors='coerce')

if table_data[col].notna().sum() > 0:

numeric_columns.append(col)

except:

continue

if not numeric_columns:

return pd.DataFrame()

table_data = table_data[numeric_columns]

table_data = table_data.dropna(axis=1, how='all')

return table_data

def interpolate_to_daily(dekadal_df, farm_type):

"""Convert dekadal data to daily using cubic spline interpolation."""

if dekadal_df.empty:

return pd.DataFrame()

dekadal_df = dekadal_df.sort_index()

start_date = dekadal_df.index.min()

end_date = dekadal_df.index.max()

daily_dates = pd.date_range(start=start_date, end=end_date, freq='D')

dekadal_numeric = (dekadal_df.index - start_date).days.values

daily_numeric = (daily_dates - start_date).days.values

daily_df = pd.DataFrame(index=daily_dates)

print(f"  Interpolating {farm_type}: {len(dekadal_df)} → {len(daily_dates)} days")

successful_farms = []

for col in dekadal_df.columns:

try:

valid_mask = dekadal_df[col].notna()

if valid_mask.sum() < 2:

continue

x_vals = dekadal_numeric[valid_mask]

y_vals = dekadal_df[col][valid_mask].values

if len(x_vals) >= 4:

cs = CubicSpline(x_vals, y_vals, bc_type='natural')

interpolated = cs(daily_numeric)

else:

interpolated = np.interp(daily_numeric, x_vals, y_vals)

interpolated = np.maximum(interpolated, 0)

# Round interpolated values to 2 decimal places

interpolated = np.round(interpolated, 2)

daily_df[col] = interpolated

successful_farms.append(col)

except Exception as e:

continue

print(f"  {farm_type}: ✓ {len(successful_farms)} farms interpolated")

return daily_df

def calculate_daily_averages(daily_dfs):

"""Calculate daily average ET values for each farm type."""

avg_df = pd.DataFrame()

for farm_type, daily_df in daily_dfs.items():

if not daily_df.empty:

daily_avg = daily_df.mean(axis=1, skipna=True)

avg_df[farm_type] = daily_avg

print(f"  {farm_type}: {daily_avg.min():.2f} - {daily_avg.max():.2f} mm/day")

return avg_df

def plot_et_comparison(avg_df, period_name="Analysis Period", save_png=True):

"""Create comparison plot of ET trends across farm types."""

plt.figure(figsize=(14, 8))

colors = {'TPR': '#2E86C1', 'DSR': '#28B463', 'AWD': '#F39C12'}

line_styles = {'TPR': '-', 'DSR': '--', 'AWD': '-.'}

for farm_type in avg_df.columns:

plt.plot(avg_df.index, avg_df[farm_type],

color=colors.get(farm_type, 'black'),

linestyle=line_styles.get(farm_type, '-'),

linewidth=3,

label=f'{farm_type} (Avg: {avg_df[farm_type].mean():.2f} mm/day)',

marker='o', markersize=5, alpha=0.8)

plt.title(f'Evapotranspiration (ET) Trends Comparison\n{period_name}',

fontsize=16, fontweight='bold', pad=20)

plt.xlabel('Date', fontsize=12, fontweight='bold')

plt.ylabel('ET Value (mm/day)', fontsize=12, fontweight='bold')

plt.gca().xaxis.set_major_formatter(mdates.DateFormatter('%Y-%m-%d'))

plt.gca().xaxis.set_major_locator(mdates.WeekdayLocator(interval=2))

plt.xticks(rotation=45)

plt.grid(True, alpha=0.3, linestyle=':', linewidth=0.5)

plt.legend(loc='upper left', frameon=True, fancybox=True, shadow=True, fontsize=11)

# Statistics box

stats_text = f"""Statistics Summary:

Total Days: {len(avg_df)}

Date Range: {avg_df.index.min().strftime('%Y-%m-%d')} to {avg_df.index.max().strftime('%Y-%m-%d')}

Farm Type Comparison:"""

for farm_type in avg_df.columns:

stats_text += f"\n  {farm_type}: {avg_df[farm_type].mean():.2f} ± {avg_df[farm_type].std():.2f} mm/day"

plt.text(0.02, 0.98, stats_text, transform=plt.gca().transAxes,

verticalalignment='top', bbox=dict(boxstyle='round', facecolor='wheat', alpha=0.8),

fontsize=9, fontfamily='monospace')

plt.tight_layout()

# Save plot as PNG

if save_png:

plot_filename = 'ET_Farm_Types_Comparison_Nov2024_Apr2025.png'

plt.savefig(plot_filename, dpi=300, bbox_inches='tight', facecolor='white', edgecolor='none')

print(f"✓ Plot saved as: {plot_filename}")

plt.show()

def generate_summary_statistics(avg_df, daily_dfs):

"""Generate detailed summary statistics."""

summary_stats = []

for farm_type in avg_df.columns:

if farm_type in daily_dfs:

daily_df = daily_dfs[farm_type]

avg_values = avg_df[farm_type]

stats = {

'Farm_Type': farm_type,

'Num_Farms': len(daily_df.columns),

'Days_Analyzed': len(daily_df),

'Mean_ET': round(avg_values.mean(), 3),

'Std_ET': round(avg_values.std(), 3),

'Min_ET': round(avg_values.min(), 3),

'Max_ET': round(avg_values.max(), 3),

'Total_ET': round(avg_values.sum(), 3),

'CV_Percent': round((avg_values.std() / avg_values.mean()) * 100, 2)

}

summary_stats.append(stats)

return pd.DataFrame(summary_stats)

def main():

"""Main analysis function"""

print("🌾 ET TREND COMPARISON ANALYSIS")

print("="*60)

# Test file access

input_file = test_file_access()

if input_file is None:

return

period_name = "November 2024 - April 2025"

try:

print("\n📊 LOADING DATA...")

farm_tables = load_farm_type_tables(input_file)

if len(farm_tables) == 0:

print("❌ No valid tables found")

return

print("\n🔄 APPLYING SPLINE INTERPOLATION...")

daily_dfs = {}

for farm_type, dekadal_df in farm_tables.items():

daily_df = interpolate_to_daily(dekadal_df, farm_type)

if not daily_df.empty:

daily_dfs[farm_type] = daily_df

if len(daily_dfs) == 0:

print("❌ No daily data generated")

return

print("\n📈 CALCULATING AVERAGES...")

avg_df = calculate_daily_averages(daily_dfs)

print("\n📋 SUMMARY STATISTICS:")

summary_stats = generate_summary_statistics(avg_df, daily_dfs)

print(summary_stats.to_string(index=False))

print("\n📊 CREATING PLOT...")

plot_et_comparison(avg_df, period_name)

print("\n💾 SAVING RESULTS...")

# Save files

try:

avg_df.reset_index().to_excel('nov_april_daily_averages.xlsx', index=False)

summary_stats.to_excel('nov_april_statistics.xlsx', index=False)

with pd.ExcelWriter('nov_april_all_farms.xlsx') as writer:

for farm_type, daily_df in daily_dfs.items():

daily_df.reset_index().to_excel(writer, sheet_name=f'{farm_type}_Daily', index=False)

print("✓ Files saved successfully!")

except Exception as e:

print(f"⚠ Could not save files: {e}")

print("\n" + "="*60)

print("🎉 ANALYSIS COMPLETED!")

print("="*60)

# Key findings

print(f"\n🔍 KEY FINDINGS for {period_name}:")

mean_ets = {}

for farm_type in avg_df.columns:

mean_et = avg_df[farm_type].mean()

mean_ets[farm_type] = mean_et

print(f"• {farm_type}: {mean_et:.2f} mm/day")

if len(mean_ets) > 1:

highest_et = max(mean_ets, key=mean_ets.get)

lowest_et = min(mean_ets, key=mean_ets.get)

print(f"\n🏆 Highest ET: {highest_et} ({mean_ets[highest_et]:.2f} mm/day)")

print(f"🥉 Lowest ET: {lowest_et} ({mean_ets[lowest_et]:.2f} mm/day)")

print(f"📏 Difference: {mean_ets[highest_et] - mean_ets[lowest_et]:.2f} mm/day")

except Exception as e:

print(f"❌ Error: {str(e)}")

import traceback

traceback.print_exc()

# Run the analysis

if __name__ == "__main__":

main()

# If running in Jupyter/Colab, execute directly

main()